Work Method 12- Window Installation

(WM12-MCDC Template)

Industry Based Project (CMGT 8800)

September 20, 2018

BCIT

Faculty Advisor: Jim Turnham, P. Eng, MASc., ISO 9001 Lead Auditor

Prepared by: Behrouz Chehrehpardaz

Table of Contents

2.	SIGNATURE PAGE	3

3.	Proponent and Project Description	4

4.	PURPOSE and SCOPE	5

5.	DEFINITIONS	5

6.	RESPONSIBILITIES	6

7.	SAFETY AND ENVIRONMENT	8

8.	SUBMITTALS	9

9.	PROCEDURE	11

9.1.	General Requirements	11

9.2.	Introduction to Windows (TS1)	12

9.3.	Window Preparation (TS2)	13

9.4.	Application of Building Paper (TS3)	16

9.5.	Sill Membrane Installation (TS4)	18

9.6.	Window Installation (TS5)	21

9.7.	Weather-Strip and Sealant (TS6)	25

10.	Quality Assurance Approval	27

11.	References	27

12.	Construction Organization Chart	28

13.	Flow Chart	29

14.	Inspection and Test Plan	30

# SIGNATURE PAGE

As an Approver, with my signature, I confirm that this Work Method is the plan for construction of the work.  If the plan changes, I will inform the Originator so that the Work Method can be revised.  Alternately, I will make revisions myself and reissue to those that require copies.

As a Reviewer, my signature confirms that I have reviewed the document and any comments to the WM have been provided to the Originator and/or to the Approver.

MCDC Construction Manager

Name:                                         	   Date: ____   __      Title:  _____________________                                                       Signature: _________________

MCDC Project Manager

Name:                                         	   Date: ____   __      Title:  _____________________                                        Signature:_________________

Contractor

Name:                                         	   Date: ____   __      Title:  _____________________                                       Signature: _________________

Initial Reviewer

Name:  	    Date:                       Title:                                                                   Signature:

# Proponent and Project Description

Company Name: MC DEVELOPMENT CORP. (MCDC)

Company type of service

MCDC is a small construction company building Single Family Houses (SFHs) in North Vancouver, and the company’s vision is to be recognized as a model of quality excellence in construction.

Project Description

Under a Project Management/Design Build method, MCDC itself or on behalf of the owners manages construction projects to build new single-family houses mostly in North Vancouver.

MCDC contract out all work activities in construction stage including Window Installation.

Work Method Activity Description

This Work Method (WM) provides the required details of how the installation of a window is carried out, and it is also subject to a series of inspections, before the commencement, during the work, and after completion.

This Work Method will be used in order to ensure full compliance with MCDC’s quality policy and Quality Plan, drawings, specifications, and BC Building Codes.

Work Method Scope

This work method shall apply to the installation of all windows that has been shown in drawings of the project.

Limitation of liability:

Any organization engaged as a Contractor or Subcontractor (the Contractor) agrees to use this Work Method only under the condition that those that wrote and developed this Work Method are to be held harmless for any errors or omissions, any inaccuracies in content resulting in any damages to property or any injury to any personnel that may be involved.  It remains the sole responsibility of the Contractor to review any and all items contained in the above Work Method and to make any changes that may be required in order to satisfy any project specification or any regulatory or statutory obligation.  As well, the Contractor shall review any and all suggested methods as contained herein and shall make any changes required and shall reissue prior to commencement of construction in order to achieve the specified product or to provide a safe work site for all workers involved.  Ownership and final responsibility for the use of all Work Methods remains with the Contractor.

# PURPOSE and SCOPE

Purpose

To define the responsibilities, describe methods and documentation to be used for installing a window in MCDC’s SFH projects.

Scope

This work method applies to all activities required for installation of any window at (the address of the project). Reference Standards include:

- British Columbia (or applicable province) Building Code 2012.

Note:  Construction documents (design drawings and specifications) should be referenced as applicable and will govern over any procedure included in this document.

# DEFINITIONS

MCDC - MC Development Corp.

CM - MCDC’s Construction Manager

PM - MCDC’s Project Manager

QC - Quality Control

WM - Work Method (this document)

WP - Work Procedure

TS - Task Step

CL - Checklist

RM - Review Meeting

NCP - Nonconformity Procedure

ITP - Inspection and Test Plan

BI - Before the TS Inspection

DI - During the TS Inspection

AI – After the TS Inspection

DNV - District of North Vancouver

SWP – Safe Work Practice

SWRB – Solid Waste Removal Bylaw (DNV)

# RESPONSIBILITIES

  - Construction Manager (CM) is responsible for project scheduling, and final approving the inspections, tests, and changes. The CM is also responsible for preparation of drawings and sketches to support construction as required and all making any changes if required.

  - Project Manager (PM) is responsible for; identifying necessary resources and assigning individual responsibilities to run and monitor the quality control procedure that defined by MCDC’s QP and this WM. He is responsible for overseeing the Quality Management Plan, enforcing project construction standards, assisting the CM in the creation of work method documents by providing appropriate sequence and task definitions, executing the project, scheduling and delegation of the roles of quality control inspections, safety, environmental items and Contractor coordination.

  - The PM is accountable for the Site Superintendent’s all responsibilities as well. The PM, for each WM contemplated for use at the site, provides a review and makes changes if necessary to any clause so that it is consistent with best practice, consistent with the building code of the Province, and consistent with local conditions. Issues should be reviewed by email with the CM.

  - Site Superintendent must work well with people and is responsible for:

  - Requesting copies of subcontractor’s liability insurance and workmen’s compensation certificate.

  - Overall site activities; applying project methodology and enforcing project construction standards; organizing field staff and ensuring they perform as required; and supervising Contractors and ensuring they perform as required

  - Assisting the PM and the Contractors in the creation and execution of work plans including revisions to these plans as necessary.

  - Assisting the PM in supervision of Contractors’ work quality.

  - Working closely with and support the Contractor to identify potential risks/opportunities, discuss necessary changes, and conduct the inspections.

  - Scheduling and monitoring each workday with appropriately resources.

  - Serving as the representative of and primary contact with the PM.

  - Attending review meetings.

  - Maintaining site logs and other documents in jobsite.

  - Ensuring the jobsite safety and ensuring that safety practices are followed.

  - Trade Contractor (Contractor) refers to the company that is bound by contract to MCDC for a certain scope of work. For their scope, the Contractor is responsible for environmental control, safety controls, and quality control for self-performed work. The Contractor is responsible to write his/her Work Methods. However, if the Contractor cannot provide the required WMs, MCDC may assist, but the final WM will be reviewed, changes made to reflect project requirements, codes, laws, and resubmitted to MCDC and owned by the Contractor. The Contractor performs the work required by the contract documents and approved Work Methods to start and complete the Project and fulfill everything indicated by the contract documents. The Contractor shall perform activities described in this WM. If any revision is needed, the Contractor shall be instructed to revise and update this WM so that the WM reflects the intent and methods of the Contractor as well. The Contractor shall be fully responsible for his means and methods, and for the content of the revised WM. The Contractor shall assign a representative who will permanently attend at the job site when the job is being done. The Site Manager or the Contractor’s site representative shall ensure following the guidelines and/or Standard Specifications outline on this work method.

# SAFETY AND ENVIRONMENT

All construction activities and job procedures shall conform to

- WCB Regulations and other applicable codes, regulations and acts

- DNV Noise Regulation Bylaw (Bylaw 7188)

- DNV Environmental Protection and Preservation Bylaw (Bylaw 6515)

Before any work takes place, the PM and Site Superintendent will ensure that all operators, laborers, and Contractors have been site orientated.

Window installation work must comply with safe practices and with the requirements of the bylaw, codes and ordinances. All work process shall be fully consistent to DNV Bylaws.

- Read and fully understand ALL manufacturers’ instructions before beginning. Failure to follow proper installation and finishing instructions may result in the denial of warranty claims for operational performance problems.

- Do not work alone. Two or more people are required. Use safe lifting techniques.

- Use caution when handling glass. Broken or cracked glass can cause serious injury.

- Wear protective gear (e.g. safety glasses, gloves, ear protection, etc.).

- Operate hand/power tools safely and follow manufacturer’s operating instructions.

- Use caution when working at elevated heights.

# SUBMITTALS

The contractor submittals to MCDC:

- Contractor Quotation for doing the job described in MCDC’s RFQ package, including

  - Contract price and time (including the start time of work on site)

  - Declaration of accepting all contract terms and documents

  - Written promise to provide the required submittals (including Contractor’s Work Method and Checklists), 14 days prior to the work start

  - Documented processes and submittals to enable the PM review

  - Contractor’s initial Work Method, Checklists, and ITP for MCDC review

- The final revision of MCDC QP reviewed and confirmed by the Contractor

- Finalized WM, ITPs, Checklists, and any other documents required by the contract documents, not later than 7 days prior to the work start time, (MCDC CM written confirmation required)

- Any drawing, specs, and designing layout which is required for carrying out the work, and in order to satisfy any project specification or any regulatory or statutory obligation.

- Window manufacturer’s installation manual and a complete list of required parts and materials

- Reports that identifies the Self inspection result and scope of work, before each MCDC scheduled inspection

- Insurance and WCB coverage

All contractor submittals are stated in the Contract and include (but not limited to)

- Windows Spec Sheet (9.3.1)

- Installation drawings

- Tools and materials required:

  - Safety glasses

  - Work gloves

  - Tape measure

  - Framer’s square

  - 4' level

  - Chisel

  - Putty knife Utility knife or other cutting tools (as needed)

  - Pry bar

  - Pliers

  - Phillips hand screwdriver

  - Power driver and bits

  - Backer rod/Minimum expanding foam sealant

  - Fiberglass insulation

  - Shims

  - Caulk gun

  - Interior and exterior window grade sealant

  - Finish nails

  - 4 - #8 x 2" flat head screws (for single-hung window installation only)

  - Hammer

- Installer’s qualification proof

- Installer’s Safety Orientation sheet

- Maintenance sheet

# PROCEDURE

## General Requirements

Consult the specifications and construction drawings to determine the requirements for any aspect of the work. This Work Method is a guideline used by MCDC to describe the work process and the process of quality control by conducting the specific Inspections and relevant Checklists. The Drawings, and Specifications as well as any code and by-law are the ultimate requirements. The PM and the Contractor shall review the Work Method and make any revision (prior to each use if necessary) so that any requirements will be identified and met.

The following Task Steps (procedures) TSs are included in this Work Method:

- 9.2 Introduction to Windows (TS1)

- 9.3 Window Preparation (TS2)

- 9.4 Application of Building Paper (TS3)

- 9.5 Sill Membrane Installation (TS4)

- 9.6 Window Installation (TS5)

- 9.7 Weather-Strip and Sealant (TS6)

Each TS comes with a Checklist and each Checklist is subject to three Inspections, before, during, and after completion of the TS. Each Checklist includes several checkpoints which must be controlled and verified by the MCDC’s PM or Site Superintendent. To continue the work and proceed to next step, the Contractor must obtain the approval of PM for all Inspections. The PM will give the approval only if all Checklist’s items are checked and passed.

The Inspections and Testing shall follow the instructions described in the Inspection and Testing Plan number 12(ITP12). The PM shall review the results of the ITP and Checklists and check if the results are acceptable. The PM will communicate the acceptable results to the CM and if the results are not acceptable, the PM will communicate this issue to the CM and the Contractor to evaluate the default and issue instructions for the corrective actions.

## Introduction to Windows (TS1)

Before you begin:

- Have a look at a window to familiarize yourself with the parts of a window.

    - Head - The main horizontal member forming the top of the window.

    - Jamb - The vertical portions of the window frame that make up its sides.

    - Frame - The entire window's outside edges, or the head, jambs and sill combined.

    - Glazing - Any glass within the entire construct of the window.

    - Pane - A framed sheet of glass within a window frame.

    - Sash - Stiles and rails, which surround each pane, combine to form a single sash

    - Sill - This is the lower equivalent of the head, and protrudes from the wall slightly.

    - Muntin Bar - A muntin bar creates a boundary between individual window panes.

## Window Preparation (TS2)

Before you begin:

- Make sure all windows for the building are accounted for on site.

- Check windows for damage before installation.

| 4438 RANGER AVE., NORTH VANCOUVER |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Window | Width | Height | GLASS | OPERATION | SCREEN |
| B1 | 96 | 18 | DUAL GLAZED | FIX | NO |
| M1 | 187 | 90 | DUAL GLAZED TEMP | SLIDER | YES |
| M2 | 33 | 90 | DUAL GLAZED TEMP | FIX | NO |
| M3 | 120 | 78 | DUAL GLAZED | LEFT BS | YES |
| M4 | 114 | 78 | DUAL GLAZED TEMP | FIX | NO |
| M5 | 108 | 78 | DUAL GLAZED | LEFT BS | YES |
| U1 | 132 | 72 | DUAL GLAZED | LEFT BS | YES |
| U2 (FRENCH DOOR) | 38 | 88.5 | DUAL GLAZED TEMP | Right Out-swing | NO |
| U3 | 120 | 72 | DUAL GLAZED | LEFT BS | YES |
| U4 | 42 | 72 | DUAL GLAZED | FIX | NO |
| U5 | 120 | 72 | DUAL GLAZED | LEFT BS | YES |

    - Confirm the size of the window opening by referring to the window schedule specifications and the architectural drawings.

    - Check the level of the window opening by using an appropriate size plumb level, and make sure that all four sides of the opening are levelled and plumbed, and that the window opening is a perfect square.

    - Make sure that there are no nails sticking out past the wall sheathing around the outside and within the window opening. Clean the window opening surface of dust and debris.

|  | Checklist 12-2: Window Preparation (TS2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC Development Corp. |  | Project: | Contractor: |  |  |  |  |  |
| Number | Checkpoints |  |  | BI | DI |  |  | AI |
| 1 | DNV permit/inspection approval as required |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 2 | Review the work with ENGINEER when needed |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 3 | Has the window opening size been matched to the architectural drawings? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 4 | Have all four sides of the window been checked for level and plumb? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 5 | Are there any nails sticking out around the outside or inside the window opening? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 6 | Is the window opening surface clean? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| Quality Scores and Completion Sign-off |  |  |  |  |  |  |  |  |
| Inspection# Quality 5 4 3 2 1 Notes: On-Time 5 4 3 2 1 Notes: Sign and date*: Cell # / ID #: Signed: Date: Task has been verified complete and in compliance with contract drawings and specifications except for non-conformances and incomplete items reported above. |  |  |  |  |  |  |  |  |
| BI= Inspection Before task begin -----------DI= Inspection During task in-process --------AI= Inspection After task completed Quality Score 5 = 100% NO problems 4 = 1 minor problems 3 = Hotspot or 2-3 minor 2 = 6+ or major problems 1 = Excessive problems On-Time Score 5 = On Time 4 = Late 3 = Late by 1 day 2 = Late by 2 days 1 = Late more than 2 days Safety Score 5 = 100% NO problems 4 = 1 minor problem 3 = Hotspot or 2-3 minor 2= 4+ or major problem 1= Injury |  |  |  |  |  |  |  |  |

## Application of Building Paper (TS3)

Before you begin:

- Make sure all window opening surfaces are clean.

    - Make sure that the building paper/house wrap is overlapped correctly. For example, it should be overlapped towards the outside, from top to bottom.

    - If tears or damage occur during installation, repair with an appropriate exterior air barrier tape (e.g tuck-tape)

    - Large tears should be repaired by larger piece of building paper/house wrap, and tuck-tape at the top and the sides but not at the bottom.

    - The staples should not be used carelessly, because the least amount of penetrations through the building paper/house wrap, the better chances there is for it not leaking.

Building Paper

Paper Overlap

|  | Checklist 12-3: Application of Building Paper (TS3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC Development Corp. |  | Project: | Contractor: |  |  |  |  |  |
| Number | Checkpoints |  |  | BI | DI |  |  | AI |
| 1 | Status of previous TS inspections are approved by the PM/DNV |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 2 | Is the building paper/house wrap overlapped from top to bottom and towards the outside? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 3 | Are tears repaired properly? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 4 | Are large tears left without tape at the bottom side? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 5 | Is the building paper/house wrap secure with enough, but not too many staples? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| Quality Scores and Completion Sign-off |  |  |  |  |  |  |  |  |
| Inspection# Quality 5 4 3 2 1 Notes: On-Time 5 4 3 2 1 Notes: Sign and date*: Cell # / ID #: Signed: Date: Task has been verified complete and in compliance with contract drawings and specifications except for non-conformances and incomplete items reported above. |  |  |  |  |  |  |  |  |
| BI= Inspection Before task begin -----------DI= Inspection During task in-process --------AI= Inspection After task completed Quality Score 5 = 100% NO problems 4 = 1 minor problems 3 = Hotspot or 2-3 minor 2 = 6+ or major problems 1 = Excessive problems On-Time Score 5 = On Time 4 = Late 3 = Late by 1 day 2 = Late by 2 days 1 = Late more than 2 days Safety Score 5 = 100% NO problems 4 = 1 minor problem 3 = Hotspot or 2-3 minor 2= 4+ or major problem 1= Injury |  |  |  |  |  |  |  |  |

## Sill Membrane Installation (TS4)

Before you begin:

- The glue application should be done perfectly in order to proceed. Within a minute or two the BLUE SKIN (or equivalent) should be peeled off and then applied to the glue surface.

    - Make sure there are no air bubbles under the membrane surface. If there are any air bubbles, then they should be popped and filled with calking.

    - Make sure there are no scratches or tears on the membrane, if there is then make sure to use a larger piece of BLUE SKIN to repair deficiency.

    - The membrane on the window sill should come up 6 inches off the right and left jambs of the window, to ensure water proofing.

    - The corner should have a double layer of the blue skin, depending on the size of the window, to prevent any holes that might result from a weak spot or a damaged area.

|  | Checklist 12-4: Sill Membrane Installation (TS4) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC Development Corp. |  | Project: | Contractor: |  |  |  |  |  |
| Number | Checkpoints |  |  | BI | DI |  |  | AI |
| 1 | Status of previous TS inspections are approved by the PM/DNV |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 2 | Have any air bubbles been filled with caulking? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 3 | Are any major scratches or tears in the BLUE SKIN covered? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 4 | Does the window sill membrane reach far enough up each side to ensure waterproofing? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 5 | Is there a double layer of BLUE SKIN (or Plastic flashing corners) on the corners of the window sill? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| Quality Scores and Completion Sign-off |  |  |  |  |  |  |  |  |
| Inspection# Quality 5 4 3 2 1 Notes: On-Time 5 4 3 2 1 Notes: Sign and date*: Cell # / ID #: Signed: Date: Task has been verified complete and in compliance with contract drawings and specifications except for non-conformances and incomplete items reported above. |  |  |  |  |  |  |  |  |
| BI= Inspection Before task begin -----------DI= Inspection During task in-process --------AI= Inspection After task completed Quality Score 5 = 100% NO problems 4 = 1 minor problems 3 = Hotspot or 2-3 minor 2 = 6+ or major problems 1 = Excessive problems On-Time Score 5 = On Time 4 = Late 3 = Late by 1 day 2 = Late by 2 days 1 = Late more than 2 days Safety Score 5 = 100% NO problems 4 = 1 minor problem 3 = Hotspot or 2-3 minor 2= 4+ or major problem 1= Injury |  |  |  |  |  |  |  |  |

## Window Installation (TS5)

Before you begin:

- Read window manufacturer’s installation instructions before proceeding.

- Apply caulking to the exterior edge of the window opening. This edge will receive the window flanges when window is installed.

    - During the installation, make sure that the window assembly is levelled in all directions. In addition, add shims in the corners if needed to ensure that the window assembly is levelled accurately.

    - Place window in the rough window opening by putting the bottom of the window on the bottom sill first, and then slowly push the top of the window into the opening.

    - Centre the window in the rough opening from the inside of the building.  A small nail puller or pry bar is useful for this step.

    - Secure the window in place by hammering a nail flush to the flange (the surrounding lip of the window which fastens the window to the wall).  Check specifications for the type of nails (or screws) to be used for fastening the window in place.

    - During the nailing, ensure that the flanges are only penetrated in the holes designated for the nails. These holes typically are evenly spaced and pre-drilled and are ready for the nails application.  Check specifications for nailing pattern on the flanges.

    - Fasten any screws required on the interior side of the window with care.

    - Close and lock window, and double check window for plumb, level, and square. Diagonal measurements must be within 1/8”. Check sight lines. Check sash operation. Adjust screws and shims as needed.

|  | Checklist 12-5: Window Installation (TS5) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC Development Corp. |  | Project: | Contractor: |  |  |  |  |  |
| Number | Checkpoints |  |  | BI | DI |  |  | AI |
| 1 | Status of previous TS inspections are approved by the PM/DNV |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 2 | Are shims used appropriately to level the window? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 3 | Is the window centred in the window opening? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 4 | Are nails fastened flush to the flange with limited damage to the flange? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 5 | Have appropriate nails (or screws) been used according to specifications? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 6 | Are flanges only penetrated in the holes designated for the nails? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 7 | Have screws on the interior side of the window been fastened with care to limit damage to the window? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| Quality Scores and Completion Sign-off |  |  |  |  |  |  |  |  |
| Inspection# Quality 5 4 3 2 1 Notes: On-Time 5 4 3 2 1 Notes: Sign and date*: Cell # / ID #: Signed: Date: Task has been verified complete and in compliance with contract drawings and specifications except for non-conformances and incomplete items reported above. |  |  |  |  |  |  |  |  |
| BI= Inspection Before task begin -----------DI= Inspection During task in-process --------AI= Inspection After task completed Quality Score 5 = 100% NO problems 4 = 1 minor problems 3 = Hotspot or 2-3 minor 2 = 6+ or major problems 1 = Excessive problems On-Time Score 5 = On Time 4 = Late 3 = Late by 1 day 2 = Late by 2 days 1 = Late more than 2 days Safety Score 5 = 100% NO problems 4 = 1 minor problem 3 = Hotspot or 2-3 minor 2= 4+ or major problem 1= Injury |  |  |  |  |  |  |  |  |

## Weather-Strip and Sealant (TS6)

Before you begin:

- Make sure window is centred enough in the rough window opening to allow for backer rod installation along the top and sides of the window.

    - Apply and ensure that the weather strip (Backer rod) fills the gap. A snug coil of foam backer rod is fast to install and won't expand or stick to everything like spray foam will.

    - Seal sides and the top. Cover the backer rod with another layer of membrane which creates a tight air and water seal. But don't seal the bottom because any water that sneaks in needs a way to sneak out.

    - From inside the house, caulk the bottom, sides, and top with high-quality 50 year polyurethane caulk. (Suggest that if windows are pre-painted, utilize the same colour un-paintable caulk because it will not pick up dirt and turn dirty brown with time.)

|  | Checklist 12-6: Weather-Strip and Sealant (TS6) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC Development Corp. |  | Project: | Contractor: |  |  |  |  |  |
| Number | Checkpoints |  |  | BI | DI |  |  | AI |
| 1 | Status of previous TS inspections are approved by the PM/DNV |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 2 | Does the weather strip fill the gap around the window? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 3 | Are the sides and top of the window sealed with membrane from the outside? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 4 | Has the interior side of the window been sealed with high quality polyurethane caulking? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 5 | Do not cover outside weep holes on bottom |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 6 | No damage to the window |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 7 | Plumb, Level, Square double and final checked? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| Quality Scores and Completion Sign-off |  |  |  |  |  |  |  |  |
| Inspection# Quality 5 4 3 2 1 Notes: On-Time 5 4 3 2 1 Notes: Sign and date*: Cell # / ID #: Signed: Date: Task has been verified complete and in compliance with contract drawings and specifications except for non-conformances and incomplete items reported above. |  |  |  |  |  |  |  |  |
| BI= Inspection Before task begin -----------DI= Inspection During task in-process --------AI= Inspection After task completed Quality Score 5 = 100% NO problems 4 = 1 minor problems 3 = Hotspot or 2-3 minor 2 = 6+ or major problems 1 = Excessive problems On-Time Score 5 = On Time 4 = Late 3 = Late by 1 day 2 = Late by 2 days 1 = Late more than 2 days Safety Score 5 = 100% NO problems 4 = 1 minor problem 3 = Hotspot or 2-3 minor 2= 4+ or major problem 1= Injury |  |  |  |  |  |  |  |  |

# Quality Assurance Approval

Only if all 15 required Inspections, associated with 6 TS Checklists mentioned in this WM, are approved by the PM and the CM as OK, the PM will carry out the final Inspection and issues the written approval if the results are OK.

If the results do not match the allowable tolerances, the PM will communicate this issue to the CM who evaluates the NCs and issues instructions for the corrective actions to be taken.

Any non-conformance shall be reported through the NCR procedure described in MCDC’s QP and is applicable to any and all phases of the installation of windows.

# References

- The Handouts and QMS sample documents provided by Mr. Jim Turnham (CMGT-7246)

- Based on Behrouz Chehrehpardaz work experience

- The WMs by Andre Ekkert

- BC Building Code

- WorkSafeBC Regulations

- DNV Bylaws

# Construction Organization Chart

MCDC Board of Directors

Construct Manager/CEO

Project Manager

Site Super Intendent

Trade Contractor

# Flow Chart

Contract

Specs

Dwgs

WM/ITP

QP

END

Pre-Work WM Review Meeting

Certificate of Completion

NCP

Corrective Action

Initial Inspection

NO                          YES

Final Inspection

Passed?

Inspection

Passed?

NO/NCP

YES

Introduction to Windows

(BI & DI & AI) Inspections

Passed?

NO/NCP

(BI & DI & AI) Inspections

Passed?

YES                                                    YES

Window Preparation

NO/NCP

Weather-Strip and Sealant

(BI & DI & AI) Inspections

Passed?

NO/NCP

YES                                                    YES

(BI & DI & AI) Inspections

Passed?

Application of Building Paper

NO/NCP

(BI & DI & AI) Inspections

Passed?

NO/NCP

Window Installation

YES                                                     YES

Sill Membrane Installation

(BI & DI & AI) Inspections

Passed?

NO/NCP

# Inspection and Test Plan

| MC Development Corp |  | Inspection and Test Plan # 21 Window Installation |  |  | PM: MCDC Project Manager C: Contractor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Contractor: |  |  | Project: |  |  |  |  |  |
| # | Inspections | To Inspect Items listed in | Time of Inspection | QC by | Acceptance Criteria | H/W/D |  | Score & (lowest) | Initial | Date |
| 1 | Initial Inspection | QMP004b | Prior to any work | C | CM approval | H |  |  |  |  |
| 2 | Introduction to Windows BI | Checklist 12-1 | Prior to TS1 | C | PM Approval |  |  |  |  |  |
| 3 | Introduction to Windows DI | Checklist 12-1 | During TS1 | C | PM Approval |  |  |  |  |  |
| 4 | Introduction to Windows AI | Checklist 12-1 | After TS1 | C | PM Approval |  |  |  |  |  |
| 5 | Preparation BI | Checklist 12-2 | Prior TS2 | C | PM Approval |  |  |  |  |  |
| 6 | Preparation DI | Checklist 12-2 | During TS2 | C | PM Approval |  |  |  |  |  |
| 7 | Preparation AI | Checklist 12-2 | After TS2 | C | PM Approval |  |  |  |  |  |
| 8 | Building Paper BI | Checklist 12-3 | Before TS3 | C | PM Approval |  |  |  |  |  |
| 9 | Building Paper DI | Checklist 12-3 | During TS3 | C | PM Approval |  |  |  |  |  |
| 10 | Building Paper AI | Checklist 12-3 | After TS3 | C | PM Approval |  |  |  |  |  |
| 11 | Sill Membrane BI | Checklist 12-4 | Before TS4 | C | PM Approval |  |  |  |  |  |
| 12 | Sill Membrane DI | Checklist 12-4 | During TS4 | C | PM Approval |  |  |  |  |  |
| 13 | Sill Membrane AI | Checklist 12-4 | After TS4 | C | PM Approval |  |  |  |  |  |
| 14 | Window Installation BI | Checklist 12-5 | Before TS5 | C | PM Approval |  |  |  |  |  |
| 15 | Window Installation DI | Checklist 12-5 | During TS5 | C | PM Approval |  |  |  |  |  |
| 16 | Window Installation AI | Checklist 12-5 | After TS5 | C | PM Approval |  |  |  |  |  |
| 17 | Sealant BI | Checklist 12-6 | Before TS6 | C | PM Approval |  |  |  |  |  |
| 18 | Sealant DI | Checklist 12-6 | During TS6 | C | PM Approval |  |  |  |  |  |
| 19 | Sealant AI | Checklist 12-6 | After TS6 | C | PM Approval |  |  |  |  |  |
| 23 | Final Inspection | List of NCs | After Completion | PM | CM Approval |  |  |  |  |  |
| ITP Accepted by ……………………… Signature ……………………………… Date ……………. |  |  |  |  |  |  |  |  |  |  |
| (BI: Inspection Before Task Begin----DI: Inspection During Task Work----AI: Inspection After Task Finished) (W: Witnessed by CM---- H: Hold further work----D: Document) |  |  |  |  |  |  |  |  |  |  |