Work Method 18- Exterior Door Installation

(WM18-MCDC Template)

Industry Based Project (CMGT 8800)

September 20, 2018

BCIT

Faculty Advisor: Jim Turnham, P. Eng, MASc., ISO 9001 Lead Auditor

Prepared by: Behrouz Chehrehpardaz

Table of Contents

2.	SIGNATURE PAGE	3

3.	Proponent and Project Description	4

4.	PURPOSE and SCOPE	5

5.	DEFINITIONS	5

6.	RESPONSIBILITIES	6

7.	SAFETY AND ENVIRONMENT	8

8.	SUBMITTALS	8

9.	PROCEDURE	10

9.1.	General Requirements	10

9.2.	Inspection of Doors (TS1)	11

9.3.	Door Preparation (TS2)	14

9.4.	Door Installation (TS3)	17

9.5.	Secure Door in Place (TS4)	21

9.6.	Insulating the Door Unit (TS5)	25

10.	Quality Assurance Approval	27

11.	References	27

12.	Construction Organization Chart	28

13.	Flow Chart	29

14.	Inspection and Test Plan	30

# SIGNATURE PAGE

As an Approver, with my signature, I confirm that this Work Method is the plan for construction of the work.  If the plan changes, I will inform the Originator so that the Work Method can be revised.  Alternately, I will make revisions myself and reissue to those that require copies.

As a Reviewer, my signature confirms that I have reviewed the document and any comments to the WM have been provided to the Originator and/or to the Approver.

MCDC Construction Manager

Name:                                         	   Date: ____   __      Title:  _____________________                                                       Signature: _________________

MCDC Project Manager

Name:                                         	   Date: ____   __      Title:  _____________________                                        Signature:_________________

Contractor

Name:                                         	   Date: ____   __      Title:  _____________________                                       Signature: _________________

Initial Reviewer

Name:  	    Date:                       Title:                                                                   Signature:

# Proponent and Project Description

Company Name: MC DEVELOPMENT CORP. (MCDC)

Company type of service

MCDC is a small construction company building Single Family Houses (SFHs) in North Vancouver, and the company’s vision is to be recognized as a model of quality excellence in construction.

Project Description

Under a Project Management/Design Build method, MCDC itself or on behalf of the owners manages construction projects to build new single-family houses mostly in North Vancouver.

MCDC contract out all work activities in construction stage including installation of exterior doors.

Work Method Activity Description

This Work Method (WM) provides the required details of how the installation of exterior doors is carried out, and it is also subject to a series of inspections, before the commencement, during the work, and after completion.

This Work Method will be used in order to ensure full compliance with MCDC’s quality policy and Quality Plan, drawings, specifications, and BC Building Codes.

Work Method Scope

This work method shall apply to the installation of all exterior doors of the project.

Limitation of liability:

Any organization engaged as a Contractor or Subcontractor (the Contractor) agrees to use this Work Method only under the condition that those that wrote and developed this Work Method are to be held harmless for any errors or omissions, any inaccuracies in content resulting in any damages to property or any injury to any personnel that may be involved.  It remains the sole responsibility of the Contractor to review any and all items contained in the above Work Method and to make any changes that may be required in order to satisfy any project specification or any regulatory or statutory obligation.  As well, the Contractor shall review any and all suggested methods as contained herein and shall make any changes required and shall reissue prior to commencement of construction in order to achieve the specified product or to provide a safe work site for all workers involved.  Ownership and final responsibility for the use of all Work Methods remains with the Contractor.

# PURPOSE and SCOPE

Purpose: To define the responsibilities, describe methods and documentation to be used for installation of the exterior doors in MCDC’s SFH projects.

Scope: This work method applies to all activities required for installation of the exterior doors at (the address of the project). Reference Standards include:

- British Columbia (or applicable province) Building Code 2012.

Note:  Construction documents (design drawings and specifications) should be referenced as applicable and will govern over any procedure included in this document.

# DEFINITIONS

MCDC - MC Development Corp.

CM - MCDC’s Construction Manager

PM - MCDC’s Project Manager

QC - Quality Control

WM - Work Method (this document)

WP - Work Procedure

TS - Task Step

CL - Checklist

RM - Review Meeting

NCP - Nonconformity Procedure

ITP - Inspection and Test Plan

BI - Before the TS Inspection

DI - During the TS Inspection

AI – After the TS Inspection

DNV - District of North Vancouver

SWP – Safe Work Practice

SWRB – Solid Waste Removal Bylaw (DNV)

# RESPONSIBILITIES

  - Construction Manager (CM) is responsible for project scheduling, and final approving the inspections, tests, and changes. The CM is also responsible for preparation of drawings and sketches to support construction as required and all making any changes if required.

  - Project Manager (PM) is responsible for; identifying necessary resources and assigning individual responsibilities to run and monitor the quality control procedure that defined by MCDC’s QP and this WM. He is responsible for overseeing the Quality Management Plan, enforcing project construction standards, assisting the CM in the creation of work method documents by providing appropriate sequence and task definitions, executing the project, scheduling and delegation of the roles of quality control inspections, safety, environmental items and Contractor coordination.

  - The PM is accountable for the Site Superintendent’s all responsibilities as well. The PM, for each WM contemplated for use at the site, provides a review and makes changes if necessary to any clause so that it is consistent with best practice, consistent with the building code of the Province, and consistent with local conditions. Issues should be reviewed by email with the CM.

  - Site Superintendent must work well with people and is responsible for:

  - Requesting copies of subcontractor’s liability insurance and workmen’s compensation certificate.

  - Overall site activities; applying project methodology and enforcing project construction standards; organizing field staff and ensuring they perform as required; and supervising Contractors and ensuring they perform as required

  - Assisting the PM and the Contractors in the creation and execution of work plans including revisions to these plans as necessary.

  - Assisting the PM in supervision of Contractors’ work quality.

  - Working closely with and support the Contractor to identify potential risks/opportunities, discuss necessary changes, and conduct the inspections.

  - Scheduling and monitoring each workday with appropriately resources.

  - Serving as the representative of and primary contact with the PM.

  - Attending review meetings.

  - Maintaining site logs and other documents in jobsite.

  - Ensuring the jobsite safety and ensuring that safety practices are followed.

  - Trade Contractor (Contractor) refers to the company that is bound by contract to MCDC for a certain scope of work. For their scope, the Contractor is responsible for environmental control, safety controls, and quality control for self-performed work. The Contractor is responsible to write his/her Work Methods. However, if the Contractor cannot provide the required WMs, MCDC may assist, but the final WM will be reviewed, changes made to reflect project requirements, codes, laws, and resubmitted to MCDC and owned by the Contractor. The Contractor performs the work required by the contract documents and approved Work Methods to start and complete the Project and fulfill everything indicated by the contract documents. The Contractor shall perform activities described in this WM. If any revision is needed, the Contractor shall be instructed to revise and update this WM so that the WM reflects the intent and methods of the Contractor as well. The Contractor shall be fully responsible for his means and methods, and for the content of the revised WM. The Contractor shall assign a representative who will permanently attend at the job site when the job is being done. The Site Manager or the Contractor’s site representative shall ensure following the guidelines and/or Standard Specifications outline on this work method.

# SAFETY AND ENVIRONMENT

All construction activities and job procedures shall conform to

- WCB Regulations and other applicable codes, regulations and acts

- DNV Noise Regulation Bylaw (Bylaw 7188)

- DNV Environmental Protection and Preservation Bylaw (Bylaw 6515)

Before any work takes place, the PM and Site Superintendent will ensure that all operators, laborers, and Contractors have been site orientated.

Exterior door installation procedures must comply with safe practices and with the requirements of the bylaw, codes and ordinances.

All work process shall be fully consistent to DNV Bylaws.

- Read and fully understand ALL manufacturers’ instructions before beginning. Failure to follow proper installation and finishing instructions may result in the denial of warranty claims for operational performance problems.

- Do not work alone. Two or more people are required. Use safe lifting techniques.

- Use caution when handling glass. Broken or cracked glass can cause serious injury.

- Wear protective gear (e.g. safety glasses, gloves, ear protection, etc.).

- Operate hand/power tools safely and follow manufacturer’s operating instructions.

- Use caution when working at elevated heights.

# SUBMITTALS

The contractor submittals to MCDC:

- Contractor Quotation for doing the job described in MCDC’s RFQ package, including

  - Contract price and time (including the start time of work on site)

  - Declaration of accepting all contract terms and documents

  - Written promise to provide the required submittals (including Contractor’s Work Method and Checklists), 14 days prior to the work start

  - Documented processes and submittals to enable the PM review

  - Contractor’s initial Work Method, Checklists, and ITP for MCDC review

- The final revision of MCDC QP reviewed and confirmed by the Contractor

- Finalized WM, ITPs, Checklists, and any other documents required by the contract documents, not later than 7 days prior to the work start time, (MCDC CM written confirmation required)

- Any drawing, specs, and designing layout which is required for carrying out the work, and in order to satisfy any project specification or any regulatory or statutory obligation.

- Reports that identifies the Self inspection result and scope of work, before each MCDC scheduled inspection

- Insurance and WCB coverage

All contractor submittals are stated in the Contract and include (but not limited to)

- Materials

  - Box Wood Shims

  - One box 21⁄2" - 3" Galvanized Screws

  - #10 x 21⁄2" Security Screws for Hinge to Jamb

  - #8 x 21⁄2" Strike Plate Screws

  - Exterior Caulk (Paintable Silicone Recommended)

  - Lockset Adjustable Strike Plates

  - Fiberglass Insulation

  - Polyurethane low expansion foam

  - Plastic drain screen with crisscross or woven pattern for step-down landings

  - Drip cap if required

  - 4" to 6" wide self-adhesive flashing and compatible primer if required (Check the local code)

- Tools

  - Safety Glasses and Gloves

  - Measuring Tape, Utility Knife, Hammer, Level, Carpenter's Square

  - Cutting Shears (sill pan)

  - Hacksaw or Reciprocating Saw,

  - Caulking Gun

  - Drill & Drill bits

  - Stiff Putty Knife

# PROCEDURE

## General Requirements

Consult the specifications and construction drawings to determine the requirements for any aspect of the work. This Work Method is a guideline used by MCDC to describe the work process and the process of quality control by conducting the specific Inspections and relevant Checklists. The Drawings, and Specifications as well as any code and by-law are the ultimate requirements. The PM and the Contractor shall review the Work Method and make any revision (prior to each use if necessary) so that any requirements will be identified and met.

The following Task Steps (procedures) TSs are included in this Work Method:

- 9.2 Inspection of Doors (TS1)

- 9.3 Door Preparation (TS2)

- 9.4 Door Installation (TS3)

- 9.5 Secure Door in Place (TS4)

- 9.6 Insulating the Door Unit (TS5)

Each TS comes with a Checklist and each Checklist is subject to three Inspections, before, during, and after completion of the TS. Each Checklist includes several checkpoints which must be controlled and verified by the MCDC’s PM or Site Superintendent. To continue the work and proceed to next step, the Contractor must obtain the approval of PM for all Inspections. The PM will give the approval only if all Checklist’s items are checked and passed.

The Inspections and Testing shall follow the instructions described in the Inspection and Testing Plan number 18(ITP18). The PM shall review the results of the ITP and Checklists and check if the results are acceptable. The PM will communicate the acceptable results to the CM and if the results are not acceptable, the PM will communicate this issue to the CM and the Contractor to evaluate the default and issue instructions for the corrective actions.

## Inspection of Doors (TS1)

    - All doors must be inspected thoroughly by the installer before installation. The manufacturers do not warrant doors installed with a defect that was obvious prior to installation.  All doors should be inspected for freight damage upon receipt and before forwarding. Any observed damage must be noted on the delivery receipt and the manufacturer should be notified immediately. Failure to follow this procedure may lead to claim denial by the shipper and/or carrier.

    - It is not recommended to deliver or install doors until building is enclosed, wet work is complete and HVAC system is operating and maintaining temperature and relative humidity at occupancy levels during the remainder of the construction period.  Recommended conditions are 25-55% Relative humidity and 50-90 Degrees Fahrenheit.  Humidity levels over or under the ones specified here may cause permanent damage and will void the manufacturer’s door warranty.

    - Do not store doors in damp areas or in freshly plastered buildings. Storage space should be dry and well ventilated. If the doors are to be stored more than a few days, all edges should be sealed with a non-water-based Sealer or primer. Stack doors flat on carefully levelled supports.

|  | Checklist 18-1: Inspection of Doors (TS1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC Development Corp. |  | Project: | Contractor: |  |  |  |  |  |
| Number | Checkpoints |  |  | BI | DI |  |  | AI |
| 1 | DNV permit/inspection approval as required |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 2 | Have the door opening sizes been matched to the architectural drawings? Location/ specifications/ and swing (handing) per plan? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 3 | Door checked for manufacturing errors? Maker/ labels and ratings per specifications |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 4 | All doors must be inspected thoroughly by the installer before installation |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 5 | Is the building enclosed, and wet work completed? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 6 | If doors are stored, is the storage space dry and well ventilated? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| Quality Scores and Completion Sign-off |  |  |  |  |  |  |  |  |
| Inspection# Quality 5 4 3 2 1 Notes: On-Time 5 4 3 2 1 Notes: Sign and date*: Cell # / ID #: Signed: Date: Task has been verified complete and in compliance with contract drawings and specifications except for non-conformances and incomplete items reported above. |  |  |  |  |  |  |  |  |
| BI= Inspection Before task begin -----------DI= Inspection During task in-process --------AI= Inspection After task completed Quality Score 5 = 100% NO problems 4 = 1 minor problems 3 = Hotspot or 2-3 minor 2 = 6+ or major problems 1 = Excessive problems On-Time Score 5 = On Time 4 = Late 3 = Late by 1 day 2 = Late by 2 days 1 = Late more than 2 days Safety Score 5 = 100% NO problems 4 = 1 minor problem 3 = Hotspot or 2-3 minor 2= 4+ or major problem 1= Injury |  |  |  |  |  |  |  |  |

## Door Preparation (TS2)

Before you begin:

- Make sure all doors for the building are accounted for on site.

- Check doors for damage before installation.

    - For most installations, the rough framed opening (the distance between studs on the right and left and the distance between the header and the sub-floor) should be 2-1/4" wider and 3" taller than the door itself.

    - Steel doors are generally 1" shorter than wood, so the rough framed opening may be somewhat shorter. Also, most door units use a 1-1/4" high threshold which provides adequate clearance for both carpet and pad. If your threshold is higher or lower than 1-1/4", the rough framed opening may need to be adjusted.  Check the rough framed opening to be sure it is level and square and not twisted.  Minor out of level conditions can be corrected with shims during installation. If the opening is badly twisted (not plumb) this needs to be repaired before continuing.

    - If your pre-hung door is fastened in the closed position with a duplex nail or removable plastic plug, remove these along with the packaging and crating. NOTE: If your door unit has pre-hanging clips, do not remove them until instructed to do so. These clips hold the door system aligned and closed during installation.

    - Put a generous bead of caulk along the outside edge of the sub-floor and another generous bead of caulk 1/2" in from the first. Be sure caulking beads seal the threshold and sub-floor to prevent the infiltration of moisture.

|  | Checklist 18-2: Door Preparation (TS2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC Development Corp. |  | Project: | Contractor: |  |  |  |  |  |
| Number | Checkpoints |  |  | BI | DI |  |  | AI |
| 1 | Status of previous TS inspections are approved by the PM/DNV |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 2 | Have the door opening sizes been matched to the architectural drawings? Location/ specifications/ and swing (handing) per plan? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 3 | Door checked for manufacturing errors? Maker/ labels and ratings per specifications |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 4 | Have all four sides of the door been checked for level? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 5 | Are there any nails sticking out around the outside of the opening? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 6 | Header and jambs: plumb/ square and level |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 7 | Is the door opening surface clean? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| Quality Scores and Completion Sign-off |  |  |  |  |  |  |  |  |
| Inspection# Quality 5 4 3 2 1 Notes: On-Time 5 4 3 2 1 Notes: Sign and date*: Cell # / ID #: Signed: Date: Task has been verified complete and in compliance with contract drawings and specifications except for non-conformances and incomplete items reported above. |  |  |  |  |  |  |  |  |
| BI= Inspection Before task begin -----------DI= Inspection During task in-process --------AI= Inspection After task completed Quality Score 5 = 100% NO problems 4 = 1 minor problems 3 = Hotspot or 2-3 minor 2 = 6+ or major problems 1 = Excessive problems On-Time Score 5 = On Time 4 = Late 3 = Late by 1 day 2 = Late by 2 days 1 = Late more than 2 days Safety Score 5 = 100% NO problems 4 = 1 minor problem 3 = Hotspot or 2-3 minor 2= 4+ or major problem 1= Injury |  |  |  |  |  |  |  |  |

## Door Installation (TS3)

Before you begin:

- Make sure all door opening surfaces are clean.

    - Apply a bead of caulk to the back for the brickmould on the door. It should be applied along the two sides and the top.

    - From the outside, tilt the entire pre-hung door unit into the center of the opening. Set the bottom of the door in the opening and tilt the top into place. All doors should be handled with the proper care to avoid any damage.

    - Make sure the sill is in contact with the caulking and that the hinge jamb is level.  Temporarily fasten the jamb in place with two 10d finish nails (or two #8 x 3" drywall screws if the jamb is to be painted instead of stained). Do not set nails. Place one nail or screw (centered in the jamb) approximately 2" above the bottom hinge and one 2" below the top hinge. Loosely fasten through the jamb and into the stud.

    - From the inside, level and shim the hinge jamb. Place shims behind each hinge. Be sure the hinge jamb is level in both directions and is square in the opening.

    - Level, square and shim the latch jamb.  Place shims in at least three locations between the jamb and the stud. Temporarily fasten the latch jamb approximately 2" from the bottom and 2"below the top of the jamb with two 10d finish nails (or two #8 x 3" drywall screws if the jamb is to be painted).  Do not shim or fasten the top of the jamb to the header on a single door unit.

    - Remove any pre-hanging clips. Be sure the door opens freely and that the space between the door and the jamb is even on all sides. Be sure the weather-strip meets the door evenly and seals on all sides. Adjust if necessary. If adjusting is necessary, loosen or remove the latch jamb fasteners and adjust the latch jamb until there is even contact with the weather-stripping.

|  | Checklist18-3: Door Installation (TS3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC Development Corp. |  | Project: | Contractor: |  |  |  |  |  |
| Number | Checkpoints |  |  | BI | DI |  |  | AI |
| 1 | Status of previous TS inspections are approved by the PM/DNV |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 2 | Weatherstrip/ frame and saddle: continuous seal/undamaged |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 3 | Threshold/Saddle and frame secure; no movement when stressed |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 4 | Is the hinge jamb level perpendicular and parallel to the wall? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 5 | Are shims placed behind each hinge? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 6 | Are all pre-hanging clips removed? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 7 | Seal check: light/ drafts and sound/ door closed Does the weather-strip seal on all sides? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 8 | Smooth swing and latching |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 9 | Finish: frame and doors/ faces and edges: No defects/damage |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| Quality Scores and Completion Sign-off |  |  |  |  |  |  |  |  |
| Inspection# Quality 5 4 3 2 1 Notes: On-Time 5 4 3 2 1 Notes: Sign and date*: Cell # / ID #: Signed: Date: Task has been verified complete and in compliance with contract drawings and specifications except for non-conformances and incomplete items reported above. |  |  |  |  |  |  |  |  |
| BI= Inspection Before task begin -----------DI= Inspection During task in-process --------AI= Inspection After task completed Quality Score 5 = 100% NO problems 4 = 1 minor problems 3 = Hotspot or 2-3 minor 2 = 6+ or major problems 1 = Excessive problems On-Time Score 5 = On Time 4 = Late 3 = Late by 1 day 2 = Late by 2 days 1 = Late more than 2 days Safety Score 5 = 100% NO problems 4 = 1 minor problem 3 = Hotspot or 2-3 minor 2= 4+ or major problem 1= Injury |  |  |  |  |  |  |  |  |

## Secure Door in Place (TS4)

Before you begin:

- Make sure the door is installed square and level to the door frame

    - Make sure the door sweep contacts the threshold evenly. You can adjust the threshold contact by adjusting the door unit for square, if an adjustable threshold is used, by turning the adjustment screws on the threshold.

    - Re-check for smooth door operation and even spacing between the door and jamb. Secure the latch jamb in place with two more 10d finish nails or #8 x 3" drywall screws. Place the nails or screws evenly between the first two already in place and fasten through the shims and into the rough framing.

    - Install four #10 x 2-1/4" wood screws through the hinge jamb and into the rough framing; two into the top hinge in the holes closest to the weather-stripping; and one each into the other hinges in the top hole closest to the weather-stripping. You may need to remove the screws supplied with the hinges to insert these longer #10 x 2-1/4" screws.

    - Note: Lock sets for the doors are usually ordered and installed at a later date.  See installation instructions and template accompanying the lock sets for proper installation method.

|  | Checklist 18-4: Secure Door in Place (TS4) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC Development Corp. |  | Project: | Contractor: |  |  |  |  |  |
| Number | Checkpoints |  |  | BI | DI |  |  | AI |
| 1 | Status of previous TS inspections are approved by the PM/DNV |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 2 | Does the door sweep contact the threshold evenly? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 3 | Does the door operate smoothly through range of motion? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 4 | Are all hinges screwed in with 2 1/4" screws? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 5 | Does the weather-strip seal on all sides? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 6 | Is the door nailed off to secure it in place? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 7 | Threshold/Saddle and frame secure; no movement when stressed |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| Quality Scores and Completion Sign-off |  |  |  |  |  |  |  |  |
| Inspection# Quality 5 4 3 2 1 Notes: On-Time 5 4 3 2 1 Notes: Sign and date*: Cell # / ID #: Signed: Date: Task has been verified complete and in compliance with contract drawings and specifications except for non-conformances and incomplete items reported above. |  |  |  |  |  |  |  |  |
| BI= Inspection Before task begin -----------DI= Inspection During task in-process --------AI= Inspection After task completed Quality Score 5 = 100% NO problems 4 = 1 minor problems 3 = Hotspot or 2-3 minor 2 = 6+ or major problems 1 = Excessive problems On-Time Score 5 = On Time 4 = Late 3 = Late by 1 day 2 = Late by 2 days 1 = Late more than 2 days Safety Score 5 = 100% NO problems 4 = 1 minor problem 3 = Hotspot or 2-3 minor 2= 4+ or major problem 1= Injury |  |  |  |  |  |  |  |  |

## Insulating the Door Unit (TS5)

Before you begin:

- Make sure Threshold/Saddle and frame are strongly secure, and there is no movement when stressed.

    - Before applying casing to the interior of your door unit you will need to insulate around the frame opening.

    - To insure maximum efficiency, caulk at the following locations:

      - Where the brickmould contacts the exterior wall

      - Where the brickmould contacts the frame

      - Where the frame contacts the sill

      - At the nose of the sill, where it contacts the sill plate

|  | Checklist 18-5: Insulating the Door Unit (TS5) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC Development Corp. |  | Project: | Contractor: |  |  |  |  |  |
| Number | Checkpoints |  |  | BI | DI |  |  | AI |
| 1 | Status of previous TS inspections are approved by the PM/DNV |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 2 | Threshold/Saddle and frame are strongly secure, and there is no movement when stressed |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 3 | Are the brickmould contacts with the frame and the exterior wall caulked? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 4 | are the frame contacts with the sill caulked properly? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 5 | Are the nose of the sill contacts with the sill plate caulked? |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 6 | Make sure the corner seals are in place |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 7 | all dust and debris removed from the surface of the door |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| 8 | Re-check for smooth door operation |  |  |  |  |  |  |  |
| Comment |  |  |  |  |  |  |  |  |
| Quality Scores and Completion Sign-off |  |  |  |  |  |  |  |  |
| Inspection# Quality 5 4 3 2 1 Notes: On-Time 5 4 3 2 1 Notes: Sign and date*: Cell # / ID #: Signed: Date: Task has been verified complete and in compliance with contract drawings and specifications except for non-conformances and incomplete items reported above. |  |  |  |  |  |  |  |  |
| BI= Inspection Before task begin -----------DI= Inspection During task in-process --------AI= Inspection After task completed Quality Score 5 = 100% NO problems 4 = 1 minor problems 3 = Hotspot or 2-3 minor 2 = 6+ or major problems 1 = Excessive problems On-Time Score 5 = On Time 4 = Late 3 = Late by 1 day 2 = Late by 2 days 1 = Late more than 2 days Safety Score 5 = 100% NO problems 4 = 1 minor problem 3 = Hotspot or 2-3 minor 2= 4+ or major problem 1= Injury |  |  |  |  |  |  |  |  |

# Quality Assurance Approval

Only if all 15 required Inspections, associated with 5 TS Checklists mentioned in this WM, are approved by the PM and the CM as OK, the PM will carry out the final Inspection and issues the written approval if the results are OK.

If the results do not match the allowable tolerances, the PM will communicate this issue to the CM who evaluates the NCs and issues instructions for the corrective actions to be taken.

Any non-conformance shall be reported through the NCR procedure described in MCDC’s QP and is applicable to any and all phases of the installation of the exterior doors.

# References

- The Handouts and QMS sample documents provided by Mr. Jim Turnham (CMGT-7246)

- Based on Behrouz Chehrehpardaz work experience

- The Door Installation WM by Andre Ekkert

- BC Building Code

- WorkSafeBC Regulations

- DNV Bylaws

# Construction Organization Chart

MCDC Board of Directors

Construct Manager/CEO

Project Manager

Site Super Intendent

Trade Contractor

# Flow Chart

Contract

Specs

Dwgs

WM/ITP

QP

END

Pre-Work WM Review Meeting

Certificate of Completion

NCP

Corrective Action

Initial Inspection

NO                          YES

Final Inspection

Passed?

Inspection

Passed?

NO/NCP

YES                                                    YES

(BI & DI & AI) Inspections

Passed?

Inspection of Doors

NO/NCP

Insulating the Door Unit

(BI & DI & AI) Inspections

Passed?

NO/NCP

(BI & DI & AI) Inspections

Passed?

YES                                                    YES

Door Preparation

NO/NCP

Secure Door in Place

(BI & DI & AI) Inspections

Passed?

NO/NCP

YES                                                    YES

(BI & DI & AI) Inspections

Passed?

Door Installation

NO/NCP

# Inspection and Test Plan

| MC Development Corp |  | Inspection and Test Plan # 18 Exterior Door Installation |  |  | CM: MCDC Construction Manager PM: MCDC Project Manager C: Contractor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Contractor: |  |  | Project: |  |  |  |  |  |
| # | Inspections | To Inspect Items listed in | Time of Inspection | QC by | Acceptance Criteria | H/W/D |  | Score & (lowest) | Initials | Date |
| 1 | Initial Inspection | QMP004b | Prior to any work | C | CM approval | H |  |  |  |  |
| 2 | Door Inspection BI | Checklist 18-1 | Prior to TS1 | C | PM Approval |  |  |  |  |  |
| 3 | Door Inspection DI | Checklist 18-1 | During TS1 | C | PM Approval |  |  |  |  |  |
| 4 | Door Inspection AI | Checklist 18-1 | After TS1 | C | PM Approval |  |  |  |  |  |
| 5 | Preparation BI | Checklist 18-2 | Prior TS2 | C | PM Approval |  |  |  |  |  |
| 6 | Preparation DI | Checklist 18-2 | During TS2 | C | PM Approval |  |  |  |  |  |
| 7 | Preparation AI | Checklist 18-2 | After TS2 | C | PM Approval |  |  |  |  |  |
| 8 | Installation BI | Checklist 18-3 | Before TS3 | C | PM Approval |  |  |  |  |  |
| 9 | Installation DI | Checklist 18-3 | During TS3 | C | PM Approval |  |  |  |  |  |
| 10 | Installation AI | Checklist 18-3 | After TS3 | C | PM Approval |  |  |  |  |  |
| 11 | Secure in Place BI | Checklist 18-4 | Before TS4 | C | PM Approval |  |  |  |  |  |
| 12 | Secure in Place DI | Checklist 18-4 | During TS4 | C | PM Approval |  |  |  |  |  |
| 13 | Secure in Place AI | Checklist 18-4 | After TS4 | C | PM Approval |  |  |  |  |  |
| 14 | Insulation BI | Checklist 18-5 | Before TS5 | C | PM Approval |  |  |  |  |  |
| 15 | Insulation DI | Checklist 18-5 | During TS5 | C | PM Approval |  |  |  |  |  |
| 16 | Insulation AI | Checklist 18-5 | After TS5 | C | PM Approval |  |  |  |  |  |
| 23 | Final Inspection | List of NCs | After Completion | PM | CM Approval |  |  |  |  |  |
| ITP Accepted by ……………………… Signature ……………………………… Date ……………. |  |  |  |  |  |  |  |  |  |  |
| (BI: Inspection Before Task Begin----DI: Inspection During Task Work----AI: Inspection After Task Finished) (W: Witnessed by CM---- H: Hold further work----D: Document) |  |  |  |  |  |  |  |  |  |  |